Object Type:    efield

Description:    An extracellular field potential recording electrode that uses

current sources and their distance from the electrode site to

calculate the field.

Author:         M. Wilson, Caltech (2/89)

------------------------------------------------------------------------------

ELEMENT PARAMETERS

DataStructure:  efield_type  [in src/device/dev_struct.h]

Size:           64 bytes

Fields:         scale

field

x

y

z

------------------------------------------------------------------------------

SIMULATION PARAMETERS

Function:       ExtracellularRecordingElectrode  [in src/device/efield.c]

Classes:        device

Actions:        PROCESS

RESET

RECALC

Messages:       CURRENT current 0.0

------------------------------------------------------------------------------

Notes:          If one assumes that the medium surrounding neurons is

of homogeneous resistivity and has no capacitance,

then the field potential generated by a compartmental

model can be calculated from the following equation:

Im

1      n	    i

F = ------  SUM  -----  (see Nunez, 1981)

4*pi*s  i=1    R

i

where F is the field potential in volts, s is

conductivity in 1/(Ohms*m), Im_i is the transmembrane

current (Amperes) accoss the ith  neural compartment,

and R_i is the distance from the ith neural

compartment to the recording electrode (i.e. the

efield object).

In the case of the efield object, the term 1/(4*pi*s)

is replaced by the scale field, otherwise the same

equation is used to calculate field potentials.

The RECALC action must be called in order for efield

to calculate the distances to neural compartments.

For more information about field potentials and how

they are used in GENESIS simulations, see Chapter 9 in

The Book of GENESIS.  For a derivation of the above

equation, see:

Nunez, P.L. (1981) Electric Fields of the Brain: The

Neurophysics of EEG, Oxford University Press, Oxford.

Example:	create efield electrode

setfield electrode scale -1e3 x 0.00 y 0.00 x 0.00

addmsg some_compartment electrode CURRENT Im 0.0

call electrode RECALC

See also:       Scripts/piriform/field.g